# Drawing-to-Inspection Sheet Automation System

## Functional Requirement Specification (Version 1.0)

### 1. Objective

Develop a software system that converts engineering drawings (PDF format) into structured inspection characteristics by combining user-assisted region definition, OCR extraction, rule-based interpretation, and customer-specific configuration.

The system shall progressively evolve from a semi-automatic solution to a fully automated drawing interpretation platform.

# 2. Scope

The system shall:

- Accept engineering drawing PDFs.

- Allow users to define logical regions within the drawing.

- Perform OCR on selected regions.

- Classify extracted information based on region type.

- Store extracted information in structured format.

- Generate inspection characteristics.

- Export inspection sheets to Excel.

- Support customer-specific interpretation rules.

The system shall not initially perform fully automatic drawing understanding.

# 3. System Philosophy

Engineering drawings are not text documents.

The meaning of text depends on:

- Location

- Context

- Symbols

- Associated dimensions

- Customer standards

Therefore:

OCR alone is insufficient.

The software shall first identify the region type and then apply region-specific processing rules.

# 4. Phase 1 – Region Definition Framework

## 4.1 Drawing Upload

User uploads:

- Native PDF

- Scanned PDF

System converts drawing into image format for processing.

## 4.2 Region Selection

User manually selects rectangular regions.

Each region shall be assigned a Region Type.

Supported Region Types:

- Drawing Area

- Notes Area

- Title Block

- Revision Table

- GD&T Area

- BOM Area

- Customer Specification Area

- General Tolerance Area

- Custom Region

System shall save:

- Region Name

- Region Type

- Coordinates

- Drawing ID

Example:

Region Name: Main Drawing

Region Type: Drawing Area

Coordinates:
X=100
Y=200
Width=800
Height=600

## 4.3 Region Storage

Each region shall be stored as:

- Coordinate definition

- Cropped image

- OCR result

- Classification metadata

# 5. Phase 2 – OCR Extraction Layer

## 5.1 OCR Processing

OCR shall run independently on each region.

OCR result shall store:

- Text

- Confidence

- X Position

- Y Position

- Width

- Height

Example:

Text = “25.00”

Confidence = 96%

Position:
X=100
Y=200

## 5.2 OCR Element Repository

Every detected text shall become an OCR Element.

Example:

Element Type:
Unknown

Text:
25.00

Location:
100,200

Confidence:
96%

# 6. Phase 3 – Region Specific Processing Rules

## 6.1 Drawing Area Processing

### Purpose

The Drawing Area contains the majority of inspection characteristics that will be converted into inspection sheet entries. Unlike Notes or Title Blocks, information in this region is highly dependent on graphical context, symbols, leader lines, and dimensional relationships.

The system shall process the Drawing Area using a configurable rule engine that identifies and classifies OCR elements into engineering characteristics.

### Processing Workflow

Step 1: OCR Extraction

Extract all text and symbols from the Drawing Area.

Examples:

- 25.00

- Ø12

- R5

- M8 x 1.25

- ±0.05

- A

- B

- C

- ⌖

- ⌀

- ⊥

Store each detected item with:

- Text value

- Position coordinates

- Bounding box

- OCR confidence

Step 2: Pattern Recognition Rules

The system shall evaluate each OCR element against configurable detection rules.

Rules shall consist of:

- Pattern Name

- Detection Expression

- Characteristic Type

- Priority

- Validation Logic

Example Rule Structure:

Rule Name:
Diameter Dimension

Pattern:
Starts with “Ø” followed by numeric value

Output Type:
Diameter

Priority:
High

### Linear Dimension Detection Rules

Purpose:

Identify standard dimensional values.

Examples:

- 25

- 50.00

- 125.5

Detection Logic:

- OCR text contains numeric value.

- Value is located near dimension lines.

- Value is not inside title block or notes region.

- Value is not part of a revision table.

Example Patterns:

- Integer number

- Decimal number

- Fractional value

Examples:

25

50.00

125.500

Output:

Characteristic Type:
Linear Dimension

Nominal Value:
Extracted numeric value

### Diameter Detection Rules

Purpose:

Identify cylindrical feature dimensions.

Detection Logic:

- OCR text begins with diameter symbol (Ø).

- Numeric value follows symbol.

- Optional tolerance may be attached.

Examples:

Ø10

Ø25 ±0.02

Ø50 +0.01/-0.01

Output:

Characteristic Type:
Diameter

Nominal Value:
25

Tolerance:
Parsed separately

### Radius Detection Rules

Purpose:

Identify arc and radius dimensions.

Detection Logic:

- OCR text begins with “R”.

- Numeric value follows.

- Located near curved geometry or leader line.

Examples:

R2

R5.5

R25 ±0.1

Output:

Characteristic Type:
Radius

### Angular Dimension Detection Rules

Purpose:

Identify angular measurements.

Detection Logic:

- OCR text contains degree symbol.

- Numeric value precedes degree symbol.

Examples:

45°

90°

120° ±0.5°

Output:

Characteristic Type:
Angle

Unit:
Degrees

### Hole Callout Detection Rules

Purpose:

Identify drilled, reamed, or counterbored holes.

Detection Logic:

- Text contains diameter symbol.

- Text contains depth, quantity, counterbore, countersink, or drill notation.

- Multiple OCR elements may need to be merged into a single callout.

Examples:

4X Ø6 THRU

Ø10 DEPTH 15

Ø8 CBORE Ø12 x 5 DEEP

Output:

Characteristic Type:
Hole Feature

Additional Attributes:

- Quantity

- Diameter

- Depth

- Counterbore Diameter

- Counterbore Depth

### Thread Callout Detection Rules

Purpose:

Identify threaded features.

Detection Logic:

- Text begins with thread designation.

- Pattern matches metric or imperial thread standards.

Examples:

M8 x 1.25

M10 x 1.5-6H

1/4-20 UNC

3/8-16 UNF

Output:

Characteristic Type:
Thread

Attributes:

- Major Diameter

- Pitch

- Thread Class

- Thread Standard

### Tolerance Detection Rules

Purpose:

Identify dimensional tolerances associated with dimensions.

Detection Logic:

- Search for tolerance symbols adjacent to dimensions.

- Associate tolerance with nearest valid dimension.

- Apply precedence rules when multiple dimensions are nearby.

Examples:

25 ±0.05

25 +0.02/-0.01

25.00 ±0.005

Output:

Nominal:
25

Upper Tolerance:
0.02

Lower Tolerance:
-0.01

### Datum Detection Rules

Purpose:

Identify datum references used in GD&T.

Detection Logic:

- Single capital letter enclosed in datum frame.

- Located near feature leader line.

- Exclude title block abbreviations.

Examples:

A

B

C

Output:

Characteristic Type:
Datum Reference

Datum ID:
A

### Feature Control Frame Detection Rules

Purpose:

Identify GD&T requirements.

Detection Logic:

- Detect rectangular segmented frame.

- Extract geometric characteristic symbol.

- Extract tolerance value.

- Extract referenced datums.

Examples:

⌖ ⌀0.05 A B C

⊥ 0.02 A

∥ 0.03 A B

Output:

Characteristic Type:
GD&T

Attributes:

- GD&T Symbol

- Tolerance Value

- Material Condition Modifier

- Datum References

### Surface Finish Detection Rules

Purpose:

Identify surface roughness requirements.

Detection Logic:

- Detect surface finish symbol.

- Extract roughness value.

- Associate with nearest feature if applicable.

Examples:

Ra 1.6

Ra 3.2

Ra 0.8

Output:

Characteristic Type:
Surface Finish

Roughness Value:
1.6

Unit:
µm

### Characteristic Prioritization Rules

When multiple rules match the same OCR element, the system shall apply rule priorities.

Example:

Text:
Ø25 ±0.02

Possible Matches:

- Numeric Dimension

- Diameter

- Tolerance

Resolution:

- Diameter Rule executes first.

- Tolerance Rule extracts tolerance.

- Combined characteristic is generated.

Final Output:

Characteristic Type:
Diameter

Nominal:
25

Upper Tolerance:
0.02

Lower Tolerance:
-0.02

### Rule Configuration Requirements

All detection rules shall be configurable through an administration interface.

Each rule shall support:

- Regular Expression Pattern

- Symbol Detection Pattern

- Region Constraints

- Neighbor Search Distance

- Priority Level

- Customer-Specific Overrides

Example Configuration:

Rule Name:
Metric Thread

Regex:
M(.)?x(.)?

Output:
Thread Characteristic

Priority:
90

Customer:
Global

This approach allows new drawing standards, customer conventions, and industry-specific symbols to be added without modifying application source code.

Purpose:

Identify potential inspection characteristics.

Detect:

- Linear dimensions

- Diameter dimensions

- Radius dimensions

- Angular dimensions

- Hole callouts

- Thread callouts

- Datum references

- Feature control frames

- Geometric tolerances

Output:

Candidate Characteristics

## 6.2 Notes Area Processing

Purpose:

Extract manufacturing and quality requirements.

Examples:

NOTE:
REMOVE ALL BURRS

NOTE:
HARDNESS 58-62 HRC

Output:

Specification Characteristics

## 6.3 Title Block Processing

Extract:

- Part Number

- Drawing Number

- Revision

- Material

- Finish

- Customer Name

- Drawing Date

Store as drawing metadata.

## 6.4 Revision Table Processing

Extract:

- Revision Level

- Revision Date

- Revision Description

Store revision history.

## 6.5 Customer Specification Area

Extract customer-specific requirements.

Examples:

Critical Characteristic

Special Characteristic

Safety Characteristic

Key Characteristic

These requirements may influence inspection frequency and control plan generation.

# 7. Characteristic Generation Engine

The system shall create structured inspection characteristics.

Example:

Input:

Ø25 ±0.02

Output:

Characteristic No: 1

Type:
Diameter

Nominal:
25

Upper Tolerance:
0.02

Lower Tolerance:
-0.02

Unit:
mm

Source Region:
Drawing Area

# 8. Customer Rule Configuration Engine

Purpose:

Allow interpretation differences between customers.

Example:

Customer A:

Triangle Symbol
= Critical Characteristic

Customer B:

Diamond Symbol
= Critical Characteristic

Customer C:

KC Symbol
= Key Characteristic

The software shall allow administrators to configure mappings.

# 9. Rule Library

Rules shall be configurable.

Examples:

Dimension Pattern

Diameter Pattern

Radius Pattern

Thread Pattern

Surface Finish Pattern

Material Pattern

Revision Pattern

Customer Characteristic Pattern

Rules shall not be hardcoded.

# 10. Inspection Sheet Generation

System shall generate:

Characteristic Number

Characteristic Description

Nominal Value

Upper Tolerance

Lower Tolerance

Unit

Characteristic Type

Criticality

Source Location

Balloon Number

Inspection Method

Remarks

# 11. Region Template Management

Users shall save templates.

Examples:

Denso Drawing Template

Mitsuba Drawing Template

Havells Drawing Template

Template contains:

- Region Definitions

- OCR Settings

- Customer Rules

Future drawings shall reuse templates.

# 12. Review and Validation Workflow

Before final generation:

System presents extracted characteristics.

User may:

- Accept

- Edit

- Delete

- Add missing characteristic

Only approved characteristics proceed to inspection sheet generation.

# 13. Future Phase (AI Assisted)

Future versions may include:

Automatic detection of:

- Title Block

- Notes

- Revision Tables

- Dimensions

- GD&T Frames

without manual region selection.

AI shall recommend classifications but user validation shall remain mandatory.

# 14. Success Criteria

Phase 1 Target:
Manual region selection + OCR extraction.

Phase 2 Target:
Automatic characteristic generation from dimensions.

Phase 3 Target:
Customer rule-based interpretation.

Phase 4 Target:
Template-driven processing.

Phase 5 Target:
AI-assisted drawing understanding with >90% extraction accuracy.

End Goal:

PDF Drawing
→ Region Classification
→ OCR
→ Rule Engine
→ Characteristic Generation
→ User Validation
→ Inspection Sheet Excel Export